APPENDIX

  - MATLAB CODES

- Basic Convotion Algorithm :

Column Inheritance:

Without image:

A = [1 5 8 2 3;1 5 4 9 2;7 4 3 6 8;1 4 5 6 2;1 6 4 5 4];

kernel = [0 -1 0;-1 5 -1;0 -1 0];

A1 = conv2(A, kernel);

A275p = A1;   A250p = A1;     A225p = A1;

for i = 1:7

for j = 1:7

% 75% Inheritance along columns

if mod(j, 4) == 0

A275p(i, j) = A1(i, j - 3);

elseif mod(j, 4) == 2

A275p(i, j) = A1(i, j - 1);

elseif mod(j, 4) == 3

A275p(i, j) = A1(i, j - 2);

end

% 50% Inheritance along columns

if mod(j, 2) == 0

A250p(i, j) = A1(i, j - 1);

end

% 25% Inheritance along columns

if mod(j, 4) == 0

A225p(i, j) = A1(i, j - 1);  end end end

disp('75% Column Inheritance:');

disp(A275p);

disp('50% Column Inheritance:');

disp(A250p);

disp('25% Column Inheritance:');

disp(A225p);

With image:

img = imread('D:/rose.jpeg');

if size(img, 3) == 3

img = rgb2gray(img);

end

A = double(img);

kernel = [1 1 1;1 1 1;1 1 1];

kernel=kernel/sum(kernel(:));

A1 = conv2(A, kernel, 'full');

A2_75 = A1;

A2_50 = A1;

A2_25 = A1;

[rows, cols] = size(A1);

for i = 1:rows

for j = 1:cols

if mod(j, 4) == 0 && j - 3 > 0

A2_75(i, j) = A1(i, j - 3);

elseif mod(j, 4) == 2 && j - 1 > 0

A2_75(i, j) = A1(i, j - 1);

elseif mod(j, 4) == 3 && j - 2 > 0

A2_75(i, j) = A1(i, j - 2);

end

Row Inheritance:

Without image:

A = [1 5 8 2 3;1 5 4 9 2;7 4 3 6 8;1 4 5 6 2;1 6 4 5 4];    kernel = [0 -1 0;-1 5 -1;0 -1 0];

A1 = conv2(A, kernel);

A275p = A1;   A250p = A1;   A225p = A1;

for i = 1:7

for j = 1:7

% 75% row Inheritance

if mod(i, 4) == 0

A275p(i, j) = A1(i - 3, j);

elseif mod(i, 4) == 2

A275p(i, j) = A1(i - 1, j);

elseif mod(i, 4) == 3

A275p(i, j) = A1(i - 2, j);

end

% 50% row Inheritance

if mod(i, 2) == 0

A250p(i, j) = A1(i - 1, j);

end

% 25% row Inheritance

if mod(i, 4) == 0

A225p(i, j) = A1(i - 1, j);end end end

disp('75% Inheritance:’);   disp(A275p);

disp('50% Inheritance:’);   disp(A250p);

disp('25% Inheritance:’);   disp(A225p);

With image:

img = imread('D:/rose.jpeg');

if size(img, 3) == 3

img = rgb2gray(img);

end

A = double(img);

kernel = [1 1 1; 1 1 1; 1 1 1];

kernel = kernel / sum(kernel(:));

A1 = conv2(A, kernel, 'full');

A2_75 = A1;

A2_50 = A1;

A2_25 = A1;

[rows, cols] = size(A1);

for i = 1:rows

for j = 1:cols

% 75% Row Inheritance

if mod(i, 4) == 0 && i - 3 > 0

A2_75(i, j) = A1(i - 3, j);

elseif mod(i, 4) == 2 && i - 1 > 0

A2_75(i, j) = A1(i - 1, j);

elseif mod(i, 4) == 3 && i - 2 > 0

A2_75(i, j) = A1(i - 2, j);

End

% 50% Row Inheritance

if mod(i, 2) == 0 && i - 1 > 0

A2_50(i, j) = A1(i - 1, j);

end

% 25% Row Inheritance

if mod(i, 4) == 0 && i - 1 > 0

A2_25(i, j) = A1(i - 1, j);

end

end

end

figure;

sgtitle("Algorithm 1- Row Inheritance");

subplot(2,2,1); imshow(uint8(A1)); title('Original Convolution');

subplot(2,2,2); imshow(uint8(A2_75)); title('75% Row Inheritance');

subplot(2,2,3); imshow(uint8(A2_50)); title('50% Row Inheritance');

subplot(2,2,4); imshow(uint8(A2_25)); title('25% Row Inheritance');

Crossing Inheritance:

Without image:

clc; clear; close all;

A = [1 5 8 2 3;1 5 4 9 2;7 4 3 6 8;1 4 5 6 2;1 6 4 5 4];

kernel = [1 1 1;1 1 1;1 1 1];

kernel=kernel/sum(kernel(:));

A1 = conv2(A, kernel);

A25 = A1;

A50 = A1;

A75 = A1;

[rows, cols] = size(A1);

for i = 2:rows

for j = 2:cols

%% 25% CROSS INHERITANCE

if mod(i, 2) == 0 && mod(j, 2) == 0

A25(i,j) = (A1(i-1,j) + A1(i,j-1))/2;

end

%%50% CROSS INHERITANCE

if mod(i+j,2)==0

A50(i,j) = (A1(i-1,j) + A1(i,j-1))/2;

end

%% 75% CROSS INHERITANCE

if mod(i, 2) == 0 && mod(j, 2) ~= 0

A75(i,j) = A1(i-1,j); % inherit from top

elseif mod(i, 2) ~= 0 && mod(j, 2) == 0

A75(i,j) = A1(i,j-1); % inherit from left

elseif mod(i,2)==0 && mod(j,2)==0

A75(i,j) = (A1(i-1,j) + A1(i,j-1))/2; % average

end

end

end

disp('75% Inheritance:'); disp(A75);

disp('50% Inheritance:'); disp(A50);

disp('25% Inheritance:'); disp(A25);

With image:

clc; clear; close all;

img = imread('D:/lena_gray.bmp');

img = double(img);

kernel = [1 1 1;1 1 1;1 1 1];

kernel=kernel/sum(kernel(:));

A1 = conv2(img, kernel);

A25 = A1;

A50 = A1;

A75 = A1;

[rows, cols] = size(A1);

for i = 2:rows

for j = 2:cols

%% 25% CROSS INHERITANCE

if mod(i, 2) == 0 && mod(j, 2) == 0

A25(i,j) = (A1(i-1,j) + A1(i,j-1))/2;

end

%%50% CROSS INHERITANCE

if mod(i+j,2)==0

A50(i,j) = (A1(i-1,j) + A1(i,j-1))/2;

End

- Matrix Rebuild Algorithm:

Without image:

clc; clear all; close all;

%% Input 3x3 Matrices

a = [1 2 0;1 1 3;0 2 2];

b = [0 2 1;0 3 2;1 1 0];

c = [1 2 1;0 1 3;3 3 2];

%% Patch Extraction Using Overlapping IDX Mapping

IDX = [1 4 2 5;4 7 5 8;2 5 3 6;5 8 6 9];

d = a(IDX)'; % Channel R

f = b(IDX)'; % Channel G

e = c(IDX)'; % Channel B

disp('Extracted Patch Matrix d :'); disp(d);

disp('Extracted Patch Matrix f :'); disp(f);

disp('Extracted Patch Matrix e :'); disp(e);

%% Input Features Matrix g (Each row is a flattened patch)

g = horzcat(d, f, e);

disp('Input Features Matrix g:'); disp(g);

%% Kernel Matrices (flattened)

k1 = [1 1; 2 2]; k2 = [1 1; 1 1]; k3 = [0 1; 1 0]; 
% Flatten each kernel in raster order and stack

h = k1([1 3 2 4])';i = k2([1 3 2 4])';j = k3([1 3 2 4])';

k = vertcat(h, i, j);

% Additional kernel set

k4 = [1 0; 0 1];k5 = [2 1; 2 1];k6 = [1 2; 2 0];

m = k4([1 3 2 4])';n = k5([1 3 2 4])';o = k6([1 3 2 4])';

p = vertcat(m, n, o);

%% Full Kernel Matrix q

q = horzcat(k, p);

disp('Kernel Matrix q:'); disp(q);

%% Matrix Multiplication Result (Simulated Convolution)

r = g * q;

disp('Initial Output Matrix r:'); disp(r);

%% Rebuild the Removed Rows by Inheriting from Row Above

u = r; % Copy of r

for i = 1:size(r, 1)

if mod(i, 3) == 2 && i > 1

u(i, :) = r(i - 1, :);

end

end

disp("Rebuilded output feature matrix:")

disp(u);

With image:

clc; clear all; close all;

img = imread('D:\brain_tumor_dataset\yes\Y11.jpg');

A = double(img);

kernel = ones(3, 3) / 9;

A1 = conv2(A, kernel, 'valid');  % 510x510

% Initialize inherited versions

A2_25 = A1;

A2_50 = A1;

A2_75 = A1;

[rows, cols] = size(A1);

% Apply inheritance logic

for i = 1:rows

for j = 1:cols

% 75% Inheritance

if mod(i, 4) == 0 && i - 3 > 0

A2_75(i, j) = A1(i - 3, j);

elseif mod(i, 4) == 2 && i - 1 > 0

A2_75(i, j) = A1(i - 1, j);

elseif mod(i, 4) == 3 && i - 2 > 0

A2_75(i, j) = A1(i - 2, j);

end

% 50% Inheritance

if mod(i, 2) == 0 && i - 1 > 0

A2_50(i, j) = A1(i - 1, j);

end

% 25% Inheritance

if mod(i, 4) == 0 && i - 1 > 0

A2_25(i, j) = A1(i - 1, j);

end

end

end

% Total patches = (510 - 2)^2 = 258064

num_patches = (rows - 2) * (cols - 2);

IDX = zeros(num_patches, 9);

index_matrix = reshape(1:rows * cols, rows, cols);

k = 1;

for j = 1:(cols - 2)

for i = 1:(rows - 2)

block = index_matrix(i:i+2, j:j+2);

IDX(k, :) = block(:)';

k = k + 1;

end

end

% Extract 3×3 patches from each matrix

patch25 = A2_25(IDX);  % size: 258064 × 9

patch50 = A2_50(IDX);

patch75 = A2_75(IDX);

% Final concatenated output: 258064 × 27

G = [patch25, patch50, patch75];

disp("Final patch matrix G (258064 × 27):");

disp(G);

% Define kernels

k1 = [-1 -1 -1; -1 8 -1; -1 -1 -1];

k2 = [ 0 -1  0; -1 5 -1;  0 -1  0];

k3 = [0.11111111111 0.11111111111 0.11111111111;0.11111111111 0.11111111111 0.11111111111;0.11111111111 0.11111111111 0.11111111111];

col1 = [k1(:); k2(:); k3(:)];

col2 = col1(randperm(27));

col3 = col1(randperm(27));

kem = [col1, col2, col3];

disp("Kernel matrix")

disp(kem);

X= G*kem;

disp("Output feature matrix");

disp(X);

u = X;  % Copy of X

for i = 1:size(X, 1)

if mod(i, 3) == 2 && i > 1

u(i, :) = X(i - 1, :);

end

end

disp("Rebuilded output feature matrix:")

disp(u);

u_img = reshape(mean(u, 2), [508, 508]);  % Average over 3 features

imshow(mat2gray(u_img)); title('Rebuilded output Feature Image');

- Adaptive Threshold Algorithm:

Without image:

A = [1 5 8 2 3;1 5 4 9 2;7 4 3 6 8;1 4 5 6 2;1 6 4 5 4];

kernel = [-1 1 1;-2 2 2;-1 1 1];

A1 = conv2(A, kernel);

T = adaptthresh(A1, 0.2);

A2 = A1;

[rows, cols] = size(A1);

for i = 2:rows

for j = 2:cols

c = mod(A1(i,j) - A1(i-1,j), j);

if c < T(i,j)

A2(i,j) = A1(i, j-1);

else

A2(i,j) = A1(i,j);

end  end   end

disp("Original Matrix A:");disp(A);

disp("After Convolution (A1):");disp(A1);

disp("Adaptive Threshold Matrix (T):");disp(T);

disp("Final Output Matrix (A2):");disp(A2);

With image:

img = imread(‘D:/lena_gray.jpeg');

if size(img, 3) == 3

img = rgb2gray(img);

end

A = double(img);

kernel = [1 1 1;1 1 1;1 1 1];

kernel=kernel/sum(kernel(:));

A1 = conv2(A, kernel);

T = adaptthresh(A1, 0.2);

A2 = A1;     [rows, cols] = size(A1);

for i = 2:rows

for j = 2:cols

c = mod(A1(i,j) - A1(i-1,j), j);

if c < T(i,j)

A2(i,j) = A1(i, j-1);

else

A2(i,j) = A1(i,j);

endendend

figure;

subplot(1,3,1); imshow(uint8(A)); title('Original Image');

subplot(1,3,2); imshow(uint8(A1)); title('After Convolution (A1)');

subplot(1,3,3); imshow(uint8(A2)); title('Final Output (A2)');

BRAIN TUMOR DETECTION CODES:

Sobel Operator Column Inheitance:

clc;clear all;close all;

img = imread('D:\brain_tumor_dataset\yes\Y11.jpg');

if size(img, 3) == 3

img = rgb2gray(img);

end

A = double(img);

kernel1 = [1 0 -1; 2 0 -2; 1 0 -1];

kernel2 = [1 2 1; 0 0 0; -1 -2 -1];

conv1 = conv2(A, kernel1, 'same');

conv2 = conv2(A, kernel2, 'same');

convolved_data_int = conv1.^2 + conv2.^2;

A1 = uint8(convolved_data_int > 4000) * 255;

A2_75 = A1;

A2_50 = A1;

A2_25 = A1;

[rows, cols] = size(A1);

for i = 1:rows

for j = 1:cols

if mod(j, 4) == 0 && j - 3 > 0

A2_75(i, j) = A1(i, j - 3);

elseif mod(j, 4) == 2 && j - 1 > 0

A2_75(i, j) = A1(i, j - 1);

elseif mod(j, 4) == 3 && j - 2 > 0

A2_75(i, j) = A1(i, j - 2);

end

if mod(j, 2) == 0 && j - 1 > 0

A2_50(i, j) = A1(i, j - 1);

end

if mod(j, 4) == 0 && j - 1 > 0

A2_25(i, j) = A1(i, j - 1);

end

end

end

figure;

sgtitle("Sobel Operator - Column Inheritance");

subplot(2,2,1); imshow(A1); title('Original Convolution (Thresholded)');

subplot(2,2,2); imshow(A2_75); title('75% Column Inheritance');

subplot(2,2,3); imshow(A2_50); title('50% Column Inheritance');

subplot(2,2,4); imshow(A2_25); title('25% Column Inheritance');

MORPHOLOGICAL DETECTION CODE:

img = imread('D:\brain_tumor_dataset\yes\Y1.jpg');

gray = rgb2gray(img);

% Enhance contrast

enhanced = imadjust(gray);

% Sharpening convolution

kernel = [-1 -1 -1; -1 9 -1; -1 -1 -1];

sharpened = imfilter(enhanced, kernel, 'same');

% Threshold

tumor_mask = imbinarize(sharpened, 0.72);

% Morphological cleanup

tumor_mask = imfill(tumor_mask, 'holes');

tumor_mask = bwareaopen(tumor_mask, 300);

tumor_mask = imclearborder(tumor_mask); %

CC = bwconncomp(tumor_mask);

if CC.NumObjects > 0

numPixels = cellfun(@numel, CC.PixelIdxList);

[~, idx] = max(numPixels);

tumor_mask(:) = 0; % reset

tumor_mask(CC.PixelIdxList{idx}) = 1;

end

% Get tumor boundary

boundaries = bwboundaries(tumor_mask);

figure;

subplot(1,4,1);

imshow(gray); title('(a) Source MRI');

subplot(1,4,2);

imshow(sharpened); title('(b) After Convolution');

subplot(1,4,3);

imshow(tumor_mask); title('(c) Segmented Tumor');

subplot(1,4,4);

imshow(gray); title('(d) Tumor Boundary Overlay');

hold on;

for k = 1:length(boundaries)

boundary = boundaries{k};

plot(boundary(:,2), boundary(:,1), 'y', 'LineWidth', 2);

end

  - VERILOG SOURCE CODES:

a)LineBuffer:

module lineBuffer(

input   i_clk,

input   i_rst,

input [7:0] i_data,

input   i_data_valid,

output [23:0] o_data,

input i_rd_data

);

reg [7:0] line [511:0]; //line buffer

reg [8:0] wrPntr;

reg [8:0] rdPntr;

always @(posedge i_clk)

begin

if(i_data_valid)

line[wrPntr] <= i_data;

end

always @(posedge i_clk)

begin

if(i_rst)

wrPntr <= 'd0;

else if(i_data_valid)

wrPntr <= wrPntr + 'd1;

end

assign o_data = {line[rdPntr],line[rdPntr+1],line[rdPntr+2]};

always @(posedge i_clk)

begin

if(i_rst)

rdPntr <= 'd0;

else if(i_rd_data)

rdPntr <= rdPntr + 'd1;

end

endmodule

b) ImageControl:

module imageControl(

input                    i_clk,

input                    i_rst,

input [7:0]              i_pixel_data,

input                    i_pixel_data_valid,

output reg [71:0]        o_pixel_data,

output                   o_pixel_data_valid,

output reg               o_intr);

reg [8:0] pixelCounter;

reg [1:0] currentWrLineBuffer;

reg [3:0] lineBuffDataValid;

reg [3:0] lineBuffRdData;

reg [1:0] currentRdLineBuffer;

wire [23:0] lb0data;

wire [23:0] lb1data;

wire [23:0] lb2data;

wire [23:0] lb3data;

reg [8:0] rdCounter;

reg rd_line_buffer;

reg [11:0] totalPixelCounter;

reg rdState;

localparam IDLE = 'b0,

RD_BUFFER = 'b1;

assign o_pixel_data_valid = rd_line_buffer;

always @(posedge i_clk)

begin

if(i_rst)

totalPixelCounter <= 0;

else

begin

if(i_pixel_data_valid & !rd_line_buffer)

totalPixelCounter <= totalPixelCounter + 1;

else if(!i_pixel_data_valid & rd_line_buffer)

totalPixelCounter <= totalPixelCounter - 1;

end

end

always @(posedge i_clk)

begin

if(i_rst)

begin

rdState <= IDLE;

rd_line_buffer <= 1'b0;

o_intr <= 1'b0;

end

else

begin

case(rdState)

IDLE:begin

o_intr <= 1'b0;

if(totalPixelCounter >= 1536)

begin

rd_line_buffer <= 1'b1;

rdState <= RD_BUFFER;

end

end

RD_BUFFER:begin

if(rdCounter == 511)

begin

rdState <= IDLE;

rd_line_buffer <= 1'b0;

o_intr <= 1'b1;

end

end

endcase

end

end

always @(posedge i_clk)

begin

if(i_rst)

pixelCounter <= 0;

else

begin

if(i_pixel_data_valid)

pixelCounter <= pixelCounter + 1;

end

end

always @(posedge i_clk)

begin

if(i_rst)

currentWrLineBuffer <= 0;

else

begin

if(pixelCounter == 511 & i_pixel_data_valid)

currentWrLineBuffer <= currentWrLineBuffer+1;

end

end

always @(*)

begin

lineBuffDataValid = 4'h0;

lineBuffDataValid[currentWrLineBuffer] = i_pixel_data_valid;

end

always @(posedge i_clk)

begin

if(i_rst)

rdCounter <= 0;

else

begin

if(rd_line_buffer)

rdCounter <= rdCounter + 1;

end

end

always @(posedge i_clk)

begin

if(i_rst)

begin

currentRdLineBuffer <= 0;

end

else

begin

if(rdCounter == 511 & rd_line_buffer)

currentRdLineBuffer <= currentRdLineBuffer + 1;

end

end

always @(*)

begin

case(currentRdLineBuffer)

0:begin

o_pixel_data = {lb2data,lb1data,lb0data};

end

1:begin

o_pixel_data = {lb3data,lb2data,lb1data};

end

2:begin

o_pixel_data = {lb0data,lb3data,lb2data};

end

3:begin

o_pixel_data = {lb1data,lb0data,lb3data};

end

endcase

end

always @(*)

begin

case(currentRdLineBuffer)

0:begin

lineBuffRdData[0] = rd_line_buffer;

lineBuffRdData[1] = rd_line_buffer;

lineBuffRdData[2] = rd_line_buffer;

lineBuffRdData[3] = 1'b0;

end

1:begin

lineBuffRdData[0] = 1'b0;

lineBuffRdData[1] = rd_line_buffer;

lineBuffRdData[2] = rd_line_buffer;

lineBuffRdData[3] = rd_line_buffer;

end

2:begin

lineBuffRdData[0] = rd_line_buffer;

lineBuffRdData[1] = 1'b0;

lineBuffRdData[2] = rd_line_buffer;

lineBuffRdData[3] = rd_line_buffer;

end

3:begin

lineBuffRdData[0] = rd_line_buffer;

lineBuffRdData[1] = rd_line_buffer;

lineBuffRdData[2] = 1'b0;

lineBuffRdData[3] = rd_line_buffer;

end

endcase

end

lineBuffer lB0(

.i_clk(i_clk),

.i_rst(i_rst),

.i_data(i_pixel_data),

.i_data_valid(lineBuffDataValid[0]),

.o_data(lb0data),

.i_rd_data(lineBuffRdData[0])

);

lineBuffer lB1(

.i_clk(i_clk),

.i_rst(i_rst),

.i_data(i_pixel_data),

.i_data_valid(lineBuffDataValid[1]),

.o_data(lb1data),

.i_rd_data(lineBuffRdData[1])

);

lineBuffer lB2(

.i_clk(i_clk),

.i_rst(i_rst),

.i_data(i_pixel_data),

.i_data_valid(lineBuffDataValid[2]),

.o_data(lb2data),

.i_rd_data(lineBuffRdData[2])

);

lineBuffer lB3(

.i_clk(i_clk),

.i_rst(i_rst),

.i_data(i_pixel_data),

.i_data_valid(lineBuffDataValid[3]),

.o_data(lb3data),

.i_rd_data(lineBuffRdData[3])

);

Endmodule

c)ImageProcessTop (top module):

module imageProcessTop1(

input   axi_clk,

input   axi_reset_n,

//slave interface

input   i_data_valid,

input [7:0] i_data,

output  o_data_ready,

//master interface

output  o_data_valid,

output [7:0] o_data,

input   i_data_ready,

//interrupt

output  o_intr

);

wire [71:0] pixel_data;

wire pixel_data_valid;

wire axis_prog_full;

wire [7:0] convolved_data;

wire convolved_data_valid;

assign o_data_ready = !axis_prog_full;

imageControl IC(

.i_clk(axi_clk),

.i_rst(!axi_reset_n),

.i_pixel_data(i_data),

.i_pixel_data_valid(i_data_valid),

.o_pixel_data(pixel_data),

.o_pixel_data_valid(pixel_data_valid),

.o_intr(o_intr)

);

conv conv(

.i_clk(axi_clk),

.i_pixel_data(pixel_data),

.i_pixel_data_valid(pixel_data_valid),

.o_convolved_data(convolved_data),

.o_convolved_data_valid(convolved_data_valid)

);

outputBuffer OB (

.wr_rst_busy(),        // output wire wr_rst_busy

.rd_rst_busy(),        // output wire rd_rst_busy

.s_aclk(axi_clk),                  // input wire s_aclk

.s_aresetn(axi_reset_n),            // input wire s_aresetn

.s_axis_tvalid(convolved_data_valid),    // input wire s_axis_tvalid

.s_axis_tready(),    // output wire s_axis_tready

.s_axis_tdata(convolved_data),      // input wire [7 : 0] s_axis_tdata

.m_axis_tvalid(o_data_valid),    // output wire m_axis_tvalid

.m_axis_tready(i_data_ready),    // input wire m_axis_tready

.m_axis_tdata(o_data),      // output wire [7 : 0] m_axis_tdata

.axis_prog_full(axis_prog_full)  // output wire axis_prog_full

);

Endmodule

d)Convolution block:

i) blurred image

module conv(

input        i_clk,

input [71:0] i_pixel_data,

input        i_pixel_data_valid,

output reg [7:0] o_convolved_data,

output reg   o_convolved_data_valid

);

integer i;

reg [7:0] kernel [8:0];

reg [15:0] multData[8:0];

reg [15:0] sumDataInt;

reg [15:0] sumData;

reg multDataValid;

reg sumDataValid;

reg convolved_data_valid;

initial

egin

for(i=0;i<9;i=i+1)

begin

kernel[i] = 1;

end

end

always @(posedge i_clk)

begin

for(i=0;i<9;i=i+1)

begin

multData[i] <= kernel[i]*i_pixel_data[i*8+:8];

end

multDataValid <= i_pixel_data_valid;

end

always @(*)

begin

sumDataInt = 0;

for(i=0;i<9;i=i+1)

begin

sumDataInt = sumDataInt + multData[i];

end

end

always @(posedge i_clk)

begin

sumData <= sumDataInt;

sumDataValid <= multDataValid;

end

always @(posedge i_clk)

begin

o_convolved_data <= sumData/9;

o_convolved_data_valid <= sumDataValid;

end

endmodule

ii) 75% Row Inheritance module: (altered to 50,25 according to algorithm)

module conv(//row

input        i_clk,

input [71:0] i_pixel_data,

input        i_pixel_data_valid,

output reg [7:0] o_convolved_data,

output reg       o_convolved_data_valid

);

integer i;

reg [7:0] kernel [8:0];

reg [15:0] multData[8:0];

reg [15:0] sumDataInt;

reg [15:0] sumData;

reg multDataValid;

reg sumDataValid;

reg [9:0] row_counter = 0;

reg [9:0] col_counter = 0;

reg [18:0] current_index;

reg [18:0] prev_index_1, prev_index_2, prev_index_3;

reg [7:0] output_memory [0:262143];

reg [7:0] inherited_pixel;

reg [7:0] final_pixel;

reg write_enable;

initial begin

for (i = 0; i < 9; i = i + 1)

kernel[i] = 1;

end

always @(posedge i_clk) begin

for (i = 0; i < 9; i = i + 1)

multData[i] <= kernel[i] * i_pixel_data[i*8 +: 8];

multDataValid <= i_pixel_data_valid;

end

always @(*) begin

sumDataInt = 0;

for (i = 0; i < 9; i = i + 1)

sumDataInt = sumDataInt + multData[i];

end

always @(posedge i_clk) begin

sumData <= sumDataInt;

sumDataValid <= multDataValid;

end

always @(posedge i_clk) begin

if (i_pixel_data_valid) begin

if (col_counter == 511) begin

col_counter <= 0;

if (row_counter == 511)

row_counter <= 0;

else

row_counter <= row_counter + 1;

end else begin

col_counter <= col_counter + 1;

end

end

end

always @(posedge i_clk) begin

if (sumDataValid) begin

current_index = row_counter * 512 + col_counter;

prev_index_1 = (row_counter > 0) ? ((row_counter - 1) * 512 + col_counter) : 0;

prev_index_2 = (row_counter > 1) ? ((row_counter - 2) * 512 + col_counter) : 0;

prev_index_3 = (row_counter > 2) ? ((row_counter - 3) * 512 + col_counter) : 0;

inherited_pixel = sumData / 9;

if ((row_counter % 4 == 0) && row_counter >= 3)

final_pixel = output_memory[prev_index_3];

else if ((row_counter % 4 == 2) && row_counter >= 1)

final_pixel = output_memory[prev_index_1];

else if ((row_counter % 4 == 3) && row_counter >= 2)

final_pixel = output_memory[prev_index_2];

else

final_pixel = inherited_pixel;

o_convolved_data <= final_pixel;

o_convolved_data_valid <= 1;

output_memory[current_index] <= final_pixel;

end else begin

o_convolved_data_valid <= 0;

end

end

endmodule

iii) 75% Column Inheritance (altered to 50,25% according to algorithm)

module conv(//column

input        i_clk,

input [71:0] i_pixel_data,

input        i_pixel_data_valid,

output reg [7:0] o_convolved_data,

output reg       o_convolved_data_valid

);

integer i;

reg [7:0] kernel [8:0];

reg [15:0] multData[8:0];

reg [15:0] sumDataInt;

reg [15:0] sumData;

reg multDataValid;

reg sumDataValid;

reg [9:0] row_counter = 0;

reg [9:0] col_counter = 0;

reg [18:0] current_index;

reg [18:0] prev_col_1, prev_col_2, prev_col_3;

reg [7:0] output_memory [0:262143];

reg [7:0] inherited_pixel;

reg [7:0] final_pixel;

initial begin

for (i = 0; i < 9; i = i + 1)

kernel[i] = 1;

end

// Multiply kernel with input pixels

always @(posedge i_clk) begin

for (i = 0; i < 9; i = i + 1)

multData[i] <= kernel[i] * i_pixel_data[i*8 +: 8];

multDataValid <= i_pixel_data_valid;

end

// Sum of all multiplied data

always @(*) begin

sumDataInt = 0;

for (i = 0; i < 9; i = i + 1)

sumDataInt = sumDataInt + multData[i];

end

always @(posedge i_clk) begin

sumData <= sumDataInt;

sumDataValid <= multDataValid;

end

// Update row and column counters

always @(posedge i_clk) begin

if (i_pixel_data_valid) begin

if (col_counter == 511) begin

col_counter <= 0;

if (row_counter == 511)

row_counter <= 0;

else

row_counter <= row_counter + 1;

end else begin

col_counter <= col_counter + 1;

end

end

end

// Column inheritance logic

always @(posedge i_clk) begin

if (sumDataValid) begin

current_index = row_counter * 512 + col_counter;

prev_col_1 = (col_counter > 0) ? (row_counter * 512 + (col_counter - 1)) : 0;

prev_col_2 = (col_counter > 1) ? (row_counter * 512 + (col_counter - 2)) : 0;

prev_col_3 = (col_counter > 2) ? (row_counter * 512 + (col_counter - 3)) : 0;

inherited_pixel = sumData / 9;

// Inherit from previous columns based on column_counter % 4

if ((col_counter % 4 == 0) && col_counter >= 3)

final_pixel = output_memory[prev_col_3];

else if ((col_counter % 4 == 2) && col_counter >= 1)

final_pixel = output_memory[prev_col_1];

else if ((col_counter % 4 == 3) && col_counter >= 2)

final_pixel = output_memory[prev_col_2];

else

final_pixel = inherited_pixel;

o_convolved_data <= final_pixel;

o_convolved_data_valid <= 1;

output_memory[current_index] <= final_pixel;

end else begin

o_convolved_data_valid <= 0;

end

end endmodule

iv) Adaptive Threshold Convolution:

module conv_adaptive_threshold (

input        i_clk,

input [71:0] i_pixel_data,

input        i_pixel_data_valid,

output reg [7:0] o_convolved_data,

output reg       o_convolved_data_valid

);

integer i;

reg [7:0] kernel [0:8];

reg [15:0] multData [0:8];

reg [15:0] sumData;

reg [7:0] conv_pixel;

reg [9:0] row_counter = 0;

reg [9:0] col_counter = 0;

reg [18:0] current_index;

// Memory to hold intermediate and final output

reg [7:0] output_memory [0:262143];  // For 512x512 image

reg [7:0] previous_row_pixels [0:511];

// Adaptive logic

reg [7:0] pixel_ij, pixel_i1j, pixel_ij1;

reg [7:0] final_pixel;

parameter THRESHOLD = 20;

initial begin

for (i = 0; i < 9; i = i + 1)

kernel[i] = 1;

end

always @(posedge i_clk) begin

for (i = 0; i < 9; i = i + 1)

multData[i] <= kernel[i] * i_pixel_data[i*8 +: 8];

end

always @(*) begin

sumData = 0;

for (i = 0; i < 9; i = i + 1)

sumData = sumData + multData[i];

conv_pixel = sumData / 9;  // Box blur average

end

always @(posedge i_clk) begin

if (i_pixel_data_valid) begin

if (col_counter == 511) begin

col_counter <= 0;

if (row_counter == 511)

row_counter <= 0;

else

row_counter <= row_counter + 1;

end else begin

col_counter <= col_counter + 1;

end

end

end

always @(posedge i_clk) begin

if (i_pixel_data_valid) begin

current_index = row_counter * 512 + col_counter;

pixel_ij = conv_pixel;

pixel_i1j = (row_counter > 0) ? previous_row_pixels[col_counter] : conv_pixel;

pixel_ij1 = (col_counter > 0) ? output_memory[current_index - 1] : conv_pixel;

if (((pixel_ij - pixel_i1j) % (col_counter + 1)) < THRESHOLD)

final_pixel = pixel_ij1;  // Inherit from previous column

else

final_pixel = pixel_ij;   // Use current convolved value

o_convolved_data <= final_pixel;

o_convolved_data_valid <= 1;

output_memory[current_index] <= final_pixel;

previous_row_pixels[col_counter] <= conv_pixel;

end else begin

o_convolved_data_valid <= 0;

end end endmodule

v) 75% Cross inheritance

module conv (

input             i_clk,

input             i_rst,

input      [71:0] i_pixel_data,

input             i_pixel_data_valid,

output reg [7:0]  o_convolved_data,

output reg        o_convolved_data_valid

);

integer i;

// 3x3 kernel weights (all ones for averaging)

reg [7:0] kernel[0:8];

initial begin

for (i = 0; i < 9; i = i + 1)

kernel[i] = 8'd1;

end

// Intermediate variables

reg [15:0] multData[0:8];

reg [15:0] sumDataInt;

reg [15:0] sumData;

reg        multDataValid, sumDataValid;

// Inheritance state

reg [7:0] row_buffer[0:511];  // Buffer to store previous row values (for top inheritance)

reg [7:0] left_pixel;         // Previous pixel value in current row (for left inheritance)

// Control counters

reg [9:0] row_counter = 0;

reg [9:0] col_counter = 0;

// Final inherited pixel logic

reg [7:0] conv_result;

reg [7:0] top_pixel;

reg [7:0] left_pixel_curr;

// Step 1: Multiply kernel with pixel data

always @(posedge i_clk) begin

for (i = 0; i < 9; i = i + 1)

multData[i] <= kernel[i] * i_pixel_data[i*8 +: 8];

multDataValid <= i_pixel_data_valid;

end

// Step 2: Sum the multiplied results (combinational)

always @(*) begin

sumDataInt = 0;

for (i = 0; i < 9; i = i + 1)

sumDataInt = sumDataInt + multData[i];

end

// Step 3: Store the final sum on clock edge

always @(posedge i_clk) begin

sumData      <= sumDataInt;

sumDataValid <= multDataValid;

end

// Step 4: Update row and column counters

always @(posedge i_clk) begin

if (i_pixel_data_valid) begin

if (col_counter == 511) begin

col_counter <= 0;

if (row_counter == 511)

row_counter <= 0;

else

row_counter <= row_counter + 1;

end else begin

col_counter <= col_counter + 1;

end

end

end

// Step 5: Final output with 75% crossing inheritance logic

always @(posedge i_clk) begin

if (i_rst) begin

o_convolved_data       <= 0;

o_convolved_data_valid <= 0;

left_pixel             <= 0;

end else if (sumDataValid) begin

conv_result     <= sumData / 9;

top_pixel       <= row_buffer[col_counter];

left_pixel_curr <= left_pixel;

// 75% CROSSING INHERITANCE LOGIC

if ((row_counter % 2 == 0) && (col_counter % 2 == 1))

o_convolved_data <= top_pixel;  // Inherit from top

else if ((row_counter % 2 == 1) && (col_counter % 2 == 0))

o_convolved_data <= left_pixel_curr;  // Inherit from left

else if ((row_counter % 2 == 0) && (col_counter % 2 == 0))

o_convolved_data <= (top_pixel + left_pixel_curr) >> 1;  // Average

else

o_convolved_data <= conv_result;  // Use convolved result

o_convolved_data_valid <= 1;

// Update buffers

row_buffer[col_counter] <= conv_result;

left_pixel              <= conv_result;

end else begin

o_convolved_data_valid <= 0;

end

end

endmodule

vi) 50% Cross Inheritance:

module conv (

input             i_clk,

input             i_rst,

input      [71:0] i_pixel_data,

input             i_pixel_data_valid,

output reg [7:0]  o_convolved_data,

output reg        o_convolved_data_valid

);

integer i;

reg [7:0] kernel[0:8];

initial begin

for (i = 0; i < 9; i = i + 1)

kernel[i] = 8'd1;

end

reg [15:0] multData[0:8];

reg [15:0] sumDataInt;

reg [15:0] sumData;

reg        multDataValid, sumDataValid;

reg [7:0] row_buffer[0:511];

reg [7:0] left_pixel;

reg [9:0] row_counter = 0;

reg [9:0] col_counter = 0;

reg [7:0] conv_result;

reg [7:0] top_pixel;

reg [7:0] left_pixel_curr;

always @(posedge i_clk) begin

for (i = 0; i < 9; i = i + 1)

multData[i] <= kernel[i] * i_pixel_data[i*8 +: 8];

multDataValid <= i_pixel_data_valid;

end

always @(*) begin

sumDataInt = 0;

for (i = 0; i < 9; i = i + 1)

sumDataInt = sumDataInt + multData[i];

end

always @(posedge i_clk) begin

sumData      <= sumDataInt;

sumDataValid <= multDataValid;

end

always @(posedge i_clk) begin

if (i_pixel_data_valid) begin

if (col_counter == 511) begin

col_counter <= 0;

if (row_counter == 511)

row_counter <= 0;

else

row_counter <= row_counter + 1;

end else begin

col_counter <= col_counter + 1;

end

end

end

always @(posedge i_clk) begin

if (i_rst) begin

o_convolved_data       <= 0;

o_convolved_data_valid <= 0;

left_pixel             <= 0;

end else if (sumDataValid) begin

conv_result     <= sumData / 9;

top_pixel       <= row_buffer[col_counter];

left_pixel_curr <= left_pixel;

if (((row_counter + col_counter) % 2) == 0)

o_convolved_data <= (top_pixel + left_pixel_curr) >> 1;

else

o_convolved_data <= conv_result;

o_convolved_data_valid <= 1;

row_buffer[col_counter] <= conv_result;

left_pixel <= conv_result;

end else begin

o_convolved_data_valid <= 0;

end

end

endmodule

vii) 25% Cross Inheritance:

module conv (

input             i_clk,

input             i_rst,

input      [71:0] i_pixel_data,

input             i_pixel_data_valid,

output reg [7:0]  o_convolved_data,

output reg        o_convolved_data_valid

);

integer i;

reg [7:0] kernel[0:8];

initial begin

for (i = 0; i < 9; i = i + 1)

kernel[i] = 8'd1;

end

reg [15:0] multData[0:8];

reg [15:0] sumDataInt;

reg [15:0] sumData;

reg        multDataValid, sumDataValid;

reg [7:0] row_buffer[0:511];

reg [7:0] left_pixel;

reg [9:0] row_counter = 0;

reg [9:0] col_counter = 0;

reg [7:0] conv_result;

reg [7:0] top_pixel;

reg [7:0] left_pixel_curr;

always @(posedge i_clk) begin

for (i = 0; i < 9; i = i + 1)

multData[i] <= kernel[i] * i_pixel_data[i*8 +: 8];

multDataValid <= i_pixel_data_valid;

end

always @(*) begin

sumDataInt = 0;

for (i = 0; i < 9; i = i + 1)

sumDataInt = sumDataInt + multData[i];

end

always @(posedge i_clk) begin

sumData      <= sumDataInt;

sumDataValid <= multDataValid;

end

always @(posedge i_clk) begin

if (i_pixel_data_valid) begin

if (col_counter == 511) begin

col_counter <= 0;

if (row_counter == 511)

row_counter <= 0;

else

row_counter <= row_counter + 1;

end else begin

col_counter <= col_counter + 1;

end

end

end

always @(posedge i_clk) begin

if (i_rst) begin

o_convolved_data       <= 0;

o_convolved_data_valid <= 0;

left_pixel             <= 0;

end else if (sumDataValid) begin

conv_result     <= sumData / 9;

top_pixel       <= row_buffer[col_counter];

left_pixel_curr <= left_pixel;

if ((row_counter % 2 == 0) && (col_counter % 2 == 0))

o_convolved_data <= (top_pixel + left_pixel_curr) >> 1;

else

o_convolved_data <= conv_result;

o_convolved_data_valid <= 1;

row_buffer[col_counter] <= conv_result;

left_pixel <= conv_result;

end else begin

o_convolved_data_valid <= 0;

end

end

endmodule

viii) Matrix Rebuild Algorithm (simplified form)

module conv (

input             i_clk,

input             i_rst,

input      [71:0] i_pixel_data,

input             i_pixel_data_valid,

output reg [7:0]  o_convolved_data,

output reg        o_convolved_data_valid

);

reg [7:0] p [0:8];  // 3x3 pixels

real G[0:26];       // 27-element feature vector

real F[0:2];        // Output features

real mean_out;

// Kernels

real k1[0:8];  // Edge

real k2[0:8];  // Sharpen

real k3[0:8];  // Blur

integer i;

initial begin

// Edge detection kernel

k1[0]=-1; k1[1]=-1; k1[2]=-1;

k1[3]=-1; k1[4]= 8; k1[5]=-1;

k1[6]=-1; k1[7]=-1; k1[8]=-1;

// Sharpen kernel

k2[0]= 0; k2[1]=-1; k2[2]= 0;

k2[3]=-1; k2[4]= 5; k2[5]=-1;

k2[6]= 0; k2[7]=-1; k2[8]= 0;

// Blur kernel (mean)

for (i = 0; i < 9; i = i + 1)

k3[i] = 1.0 / 9.0;

end

always @(posedge i_clk) begin

if (i_rst) begin

o_convolved_data <= 0;

o_convolved_data_valid <= 0;

end else if (i_pixel_data_valid) begin

// Extract pixels

for (i = 0; i < 9; i = i + 1)

p[i] = i_pixel_data[i*8 +: 8];

// Inheritance simulation

for (i = 0; i < 9; i = i + 1) begin

G[i]     = p[i];                            // 25%

G[i+9]   = (i > 0) ? p[i-1] : p[i];          // 50%

G[i+18]  = (i > 1) ? p[i-2] : p[i];          // 75%

end

// Feature fusion

F[0] = 0; F[1] = 0; F[2] = 0;

for (i = 0; i < 9; i = i + 1) begin

F[0] = F[0] + G[i]    * k1[i];

F[1] = F[1] + G[i+9]  * k2[i];

F[2] = F[2] + G[i+18] * k3[i];

end

mean_out = (F[0] + F[1] + F[2]) / 3.0;

// Clamp and assign output

if (mean_out > 255)

o_convolved_data <= 8'd255;

else if (mean_out < 0)

o_convolved_data <= 8'd0;

else

o_convolved_data <= mean_out;

o_convolved_data_valid <= 1'b1;

end else begin

o_convolved_data_valid <= 1'b0;

end

end

endmodule

TestBench Code:

`define headerSize 1080

`define imageSize 512*512

module tb(

);

reg clk;

reg reset;

reg [7:0] imgData;

integer file,file1,i;

reg imgDataValid;

integer sentSize;

wire intr;

wire [7:0] outData;

wire outDataValid;

integer receivedData=0;

initial

begin

clk = 1'b0;

forever

begin

#5 clk = ~clk;

end

end

initial

begin

reset = 0;

sentSize = 0;

imgDataValid = 0;

#100;

reset = 1;

#100;

file = $fopen("lena_gray.bmp","rb");

file1 = $fopen("blurred_lena.bmp","wb");

for(i=0;i<`headerSize;i=i+1)

begin

$fscanf(file,"%c",imgData);

$fwrite(file1,"%c",imgData);

end

for(i=0;i<4*512;i=i+1)

begin

@(posedge clk);

$fscanf(file,"%c",imgData);

imgDataValid <= 1'b1;

end

sentSize = 4*512;

@(posedge clk);

imgDataValid <= 1'b0;

while(sentSize < `imageSize)

begin

@(posedge intr);

for(i=0;i<512;i=i+1)

begin

@(posedge clk);

$fscanf(file,"%c",imgData);

imgDataValid <= 1'b1;

end

@(posedge clk);

imgDataValid <= 1'b0;

sentSize = sentSize+512;

end

@(posedge clk);

imgDataValid <= 1'b0;

@(posedge intr);

for(i=0;i<512;i=i+1)

begin

@(posedge clk);

imgData <= 0;

imgDataValid <= 1'b1;

end

@(posedge clk);

imgDataValid <= 1'b0;

@(posedge intr);

for(i=0;i<512;i=i+1)

begin

@(posedge clk);

imgData <= 0;

imgDataValid <= 1'b1;

end

@(posedge clk);

imgDataValid <= 1'b0;

$fclose(file);

end

always @(posedge clk)

begin

if(outDataValid)

begin

$fwrite(file1,"%c",outData);

receivedData = receivedData+1;

end

if(receivedData == `imageSize)

begin

$fclose(file1);

$stop;

end

end

imageProcessTop1 dut(

.axi_clk(clk),

.axi_reset_n(reset),

//slave interface

.i_data_valid(imgDataValid),

.i_data(imgData),

.o_data_ready(),

//master interface

.o_data_valid(outDataValid),

.o_data(outData),

.i_data_ready(1'b1),

//interrupt

.o_intr(intr)

);

endmodule